|  | Simulation of mte380_01_vertical_mount_02 Date: February 20, 2017 Designer: Solidworks Study name: Bending Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Contact Information 5 Mesh Information 6 Sensor Details 7 Resultant Forces 7 Beams 7 Study Results 8 Conclusion 10 Appendix 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: mte380_01_vertical_mount_02 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. 3/16 (0.1875) Diameter Hole1; Solid Body; Mass:0.292361 kg Volume:0.00010909 m^3 Density:2680 kg/m^3 Weight:2.86514 N; C:\Users\Mickey\Documents\UWaterloo\3rd Year\3B\MTE380\CAD\Lift vJake\mte380_01_vertical_mount_02.sldprt Feb 19 18:35:54 2017.

# Study Properties Study name; Bending. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (C:\Users\Mickey\Documents\UWaterloo\3rd Year\3B\MTE380\CAD\Lift vJake).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 5052-O. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 9e+007 N/m^2. Tensile strength:; 1.95e+008 N/m^2. Elastic modulus:; 7e+010 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2680 kg/m^3. Shear modulus:; 2.59e+010 N/m^2. Thermal expansion coefficient:; 2.38e-005 /Kelvin.; SolidBody 1(3/16 (0.1875) Diameter Hole1)(mte380_01_vertical_mount_02). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 8 edge(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.00265861; 22.1064; 9.36667; 24.0089. Reaction Moment(N.m); 0; 0; 0; 0.. <L_LdName/>; <Label_LoadImage/>; <Label_LoadDetails/>. <LoadName/>; <Image_Load/>; <Details_Load/>..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0 in. Minimum element size; 0 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 18178. Total Elements; 8603. Maximum Aspect Ratio; 7.5507. % of elements with Aspect Ratio < 3; 94.1. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:; MICKEYWUUU.

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.00265861; 22.1064; 9.36667; 24.0089. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 27547.6 N/m^2 Node: 2357; 1.95796e+007 N/m^2 Node: 968. mte380_01_vertical_mount_02-Bending-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 mm Node: 133; 0.70228 mm Node: 370. mte380_01_vertical_mount_02-Bending-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 2.84731e-007 Element: 276; 0.000128564 Element: 7662. mte380_01_vertical_mount_02-Bending-Strain-Strain1. Image-1.

# Conclusion

# Appendix